Cheshan Jayathilaka

building an Active Directory RUNNING HOME lab

Table of Contents

Set up an Active Directory home lab	1

Prerequisites:	1

Installing VirtualBox	1

Downloading Windows ISOs	1

Create the VMs	2

Create the server VM	2

Create the client VM	2

Create a VM group	2

Update VM settings	3

Other settings	3

Starting the virtual machine and installing the OS	4

Install Guest Additions	4

Rename the PC	5

Configure network settings	5

Server	5

Client	5

Install Active Directory Domain Services (AD DS)	6

Create a new administrator account	7

Install Remote Access Services and Network Address Translation (RAS/NAT)	8

RAS Installation	8

NAT Configuration	9

DHCP	9

Install Wizard	9

Configuration	10

Windows 10 client	10

Configure the Windows 10 client	10

Verify the connection from the server side	11

Lab Practices vs. Best Practices	12

Resources	12

# Set up an Active Directory home lab

In this lab, I have set up several virtual machines in VirtualBox to simulate a real-world Active Directory environment. Several online tutorials inspire the lab. I have listed them at the end of the report.

## Prerequisites:

- VirtualBox or VMware Workstation

- Windows Server 2019, 2022, or 2025 ISO. I have used the Server 2022 Desktop Experience Evaluation in this lab.

- Windows 10 ISO

## Installing VirtualBox

Download the latest stable VirtualBox x64 version. Then install the software with default settings. You may also need to download the Extension Pack to enable guest services on client VMs.

## Downloading Windows ISOs

You can download all required ISOs from the Microsoft website. To download the correct version, search for the necessary operating system, and the direct download page will be displayed on the results page.

## Create the VMs

### Create the server VM

- Run the VirtualBox software.

- Then, click on the "New" option on the right side.

- In the dialog box, I can give a suitable name to identify the server.

- Then select the correct folder to store the VM.

- I can leave the ISO location as it is since I am creating VM templates first.

- Select the operating system type as Microsoft Windows.

- Select the correct version from the dropdown. I selected Windows Server 2022 (64-bit).

- For hardware, I left the default settings as they will be sufficient for the lab.

- Additionally, the hard disk size was left at its default setting.

- Click Finish.

### Create the client VM

- Follow the same steps as above.

- In the version dropdown, select the Windows 10 (64-bit) option.

- Click Finish.

## Create a VM group

This step is optional, but it is essential to organize your VMs if you have a large number of VMs within VirtualBox.

## Update VM settings

- Right-click on the VM.

- Then click on Settings.

- Go to General and click on Advanced.

- Here, select "Bidirectional" for both the Shared Clipboard and Drag and Drop options.

- This setting will be helpful later when we need to copy something from the host machine to a client.

- I also set the boot order to Optical first and Hard Disk second, as I need to boot from the ISO for the first time to install the OS.

- After the installation, I changed this to the Hard Disk first.

- Then I set the network settings. In this area:

- Server:

  - Enable both Adapter 1 and Adapter 2.

  - Select NAT for Adapter 1 and leave the default settings.

  - For Adapter 2, select 'Internal Network' and rename it 'VMInternal' for easy identification.

- Client:

  - Only turn on Adapter 1, select 'Internal Network', and then select the network as 'VMInternal'.

## Other settings

I kept all other settings at their defaults. But if someone needs to change them, they can configure each VM individually.

## Starting the virtual machine and installing the OS

- The steps are almost identical for both the server and the client. However, the client has additional steps to customize the OS according to the user's specific needs.

- First, click on the virtual machine and turn it on.

- It will ask for a DVD to boot from since we haven't provided an ISO image yet.

- You can select the correct downloaded ISO and then click 'Mount' and 'Retry Boot'.

- Then the VM will boot into the installation wizard.

- You can go through the initial setup, and when it comes to choosing the OS to install, select:

- Server: Windows Server 2022 Evaluation Desktop Experience.

- Windows 10: Windows 10 Pro.

- Other versions do not have the feature enabled to connect to a domain. You can use Pro or Education versions.

- After the installation is complete, the OS will prompt you to create a username and password.

- Server: It will prompt you to set up a password for the Local Administrator account.

- Windows 10: You can create any username and password.

- Finish the setup.

## Install Guest Additions

- Once inside, go to This PC.

- Then, click "Devices" on the VirtualBox menu and select "Insert Guest Additions CD Image" to mount the ISO image to the DVD drive.

- Install the Guest Additions.

## Rename the PC

- Click on Settings and go to the Computer Details section, then rename the PC according to the OS.

- For example, you can rename the server PC to DC01 and the client to VMC01.

- This change can also be done via the sysdm.cpl utility. Run the command from the Run dialog.

- Click on the Change button, then rename the PC.

- Restart the VM.

## Configure network settings

### Server

- On the server, we have two Ethernet interfaces. One connects to the external network, which is connected to the router and then to the Internet. The router's DHCP server will automatically assign the IP address for this interface.

- The other interface handles the internal connection to the VirtualBox internal network.

- I used a Class B private IP address for the internal interface:

- IP Address: 172.16.0.1

- Subnet Mask: 255.255.255.0

- DNS: 127.0.0.1 (loopback address)

### Client

- The client machines only have internal network connections because we want to separate them from the outside network — they do not need to connect to the Internet directly.

- For this purpose, we configure the internal network's DNS to point to the DC's internal Ethernet IP address.

- The IP address for the clients is left to automatic because the DHCP setup on the server will automatically assign an IP address to the clients. The DHCP configuration is explained later in this article.

## Install Active Directory Domain Services (AD DS)

- Open Server Manager.

- This window is usually automatically launched when you log in to the server.

- If it does not come up, Server Manager can be launched from the Start menu.

- Once inside Server Manager, click on Add Roles and Features.

- If the option is not listed on the shortcut list, it can be found under the Manage menu.

- Once the wizard opens, select the installation type as either Role-based or Feature-based installation.

- Click Next to proceed to server selection. Once there, our server PC name and the assigned IP address will be displayed.

- Select the server.

- Under Server Roles, select Active Directory Domain Services.

- Continue through the setup wizard and complete the installation of AD DS on the server.

- Once it finishes, the Server Manager notification center will display a notification to promote the server to a domain controller, completing the installation of AD DS.

- Click on the notification, and it will open a configuration wizard.

- Since I haven't set up a previous domain or forest, I created a new forest.

(A forest is the top-most logical container in AD. Think of it as a large building with several companies operating inside. In the context of AD, each company can be viewed as a single domain, and the departments within those companies can be seen as branches of the AD. Example: Forest → company1.com and company2.com as domains. Then hr.company1.com/sales.company1.com and it.company2.com/hr.company2.com as trees.)

- Then, under the Root Domain Name field, enter a fully qualified domain name (FQDN).

- This can be something like company1.com, adhomelab.com, or, in an enterprise environment, a meaningful domain representing the actual organization.

- Click Next and, in the next window, keep the default settings.

- Add a password for Directory Services Restore Mode (DSRM).

- In the next window, wait for the wizard to generate a NETBIOS domain name.

- If needed, it can be changed here.

- Once you click Next, the wizard will check the prerequisites and inform you if any corrections are needed before promoting the server.

- If all checks pass, we can proceed with installing and completing the server promotion.

- The process will restart the server. During this, all local Administrator accounts and users will be disabled.

- You are left with the domain Administrator account until you create more administrators and users.

## Create a new administrator account

- After logging in to the server, go to Active Directory Users and Computers.

- This can be found in the Tools menu in Server Manager.

- Once opened, we can see our domain on the left pane.

- It contains all the computers and users in separate folders, and we can create new users in the Users folder.

- However, for this demonstration, I created a separate organizational unit called "_ADMINS" and added a new domain administrator to it.

- To do this, right-click on the domain name and go to New → Organizational Unit.

- Provide the name and uncheck the 'Protect container from accidental deletion' box.

- Now right-click on _ADMINS and choose New → User.

- Fill in all the necessary details.

- In the next window, provide a password and uncheck all checkboxes except 'Password never expires'.

- This is done only for demonstration purposes. In production, this is not best practice.

- Now, right-click on the user and go to Properties.

- There, you'll see an area called 'Member Of'. Click on that, and you'll see that this user is currently only a member of the Domain Users group.

- Hence, we cannot use this user to log in to the DC.

- To enable this, add Domain Admins to the membership list and click OK.

- Once finished and the user is created, we can sign off from the Administrator account and log in with the newly created user credentials.

## Install Remote Access Services and Network Address Translation (RAS/NAT)

- In this AD demo, we have a few clients that are connected only to the internal network. That means they are separated from the outside world and can't access the Internet directly. To enable them to connect to the Internet through the server safely, we need to enable RAS and NAT.

- With RAS, the clients can connect to the Internet via the local network. It will act as a router/gateway to the clients.

- NAT will translate all private IP addresses of the clients into a single public-facing IP address (of the server's internet connection) whenever they need to connect outside the local network.

### RAS Installation

- Go to the Add Roles and Features Wizard.

- Click on "Remote Access," then click "Next."

- Under Role Services, enable Routing. This also activates DirectAccess and VPN simultaneously.

- Finish the installation.

### NAT Configuration

- Click on Routing and Remote Access from the Tools menu.

- Once it opens, you'll see that the DC is currently stopped.

- Right-click on it and click Configure and Enable Routing and Remote Access.

- Select NAT from the options and click Next.

- In the next window, you will be asked to select a network interface that connects to the Internet.

- In some versions of the server, you might find that no interfaces populate here initially.

- To fix this, cancel or close the wizard and reopen it.

- Once reopened, all configured interfaces will populate in the list.

- Select the internet-facing Ethernet connection (the NAT adapter in this demo).

- Click Next and finish the setup.

- If you recheck the RAS window, you will see the DC is now in the UP state.

## DHCP

### Install Wizard

- Now we need to configure DHCP. This is where the server handles incoming IP requests from all clients in the local network.

- The server can utilize the DHCP service to lease private IP addresses from a predefined pool.

- Go to Add Roles and Features and select DHCP Server from the list.

- Finish the wizard.

### Configuration

- Open the DHCP console from the Tools menu.

- Once inside, you will see the domain name. Expand it and right-click on IPv4.

- Then create a new scope.

- In the wizard, give an easily identifiable name to the scope.

- In the range window, I used the range 172.16.0.100 to 172.16.0.200 with the subnet mask 255.255.255.0.

- During the lease duration window, I kept the default duration, but it can be reduced according to organizational policy.

- Once a client finishes its lease, it needs to either renew or release the IP.

- Then it can request a new IP when it reconnects.

- Choose the option to apply the scope immediately.

- In the next window, it will ask for a router IP — provide the DC's internal Ethernet IP (172.16.0.1) and click Add.

- The following window will request the domain name and IP address for the DNS server.

- We can keep our root domain name and set the IP address as the DC's internal Ethernet IP (172.16.0.1).

- Finish the wizard.

## Windows 10 client

### Configure the Windows 10 client

- As we already enabled a network adapter and assigned it to the VMInternal network, once we run the OS, it will recognize the domain in the local network. This is because the VMInternal network is also linked to the second Ethernet port on the DC.

- However, the Windows 10 client is still not connected to the domain, so it cannot access any resources within it.

- To do this, we first need to rename the computer and then connect the client to the domain.

- To achieve this together, open the Run dialog box, type sysdm.cpl, and press Enter.

- This will open System Properties.

- Click on the Change button.

- As we already renamed the client earlier, we only need to connect it to the domain.

- Click on the Domain option and enter the root domain address (fully qualified domain name, or FQDN).

- The client will identify this and send a connection request to the DC since it's already on the VMInternal network.

- Once you enter the domain name and click OK, it will prompt you to enter admin credentials to join the domain.

- After entering the credentials, the client will be connected to the domain.

- Then we can check the IP settings to verify whether DHCP and DNS are running as expected.

- On the client, open CMD and run the following commands:

- ipconfig /all    → Shows the client IP address from the DHCP pool

- ping 172.16.0.1  → Tests connectivity to the DC

- nslookup DOMAINNAME → Verifies DNS resolution

- ping www.google.com → Verifies internet access via RAS/NAT

- If these are working correctly, you should receive positive ping replies.

### Verify the connection from the server side

- Go to the server and open the DHCP window.

- Then, go to the scope we created and click on 'Address Leases'.

- You should be able to see the client computer on the right-hand side.

## Lab Practices vs. Best Practices

As noted in the report, this is a lab environment, so most configurations were done to keep the process simple. In a real environment, they might not be best practices.

- Password Never Expires: You correctly noted that setting "password never expires" is not a best practice. In a real environment, you would enforce a password expiration policy.

- Disabling Accidental Deletion: You unticked the "accidental deletion" box for the OU. In production, it's best to leave this checked to prevent accidental deletion of an OU containing many users or computers.

- Single Domain Controller (DC): This lab uses one DC. In a real-world environment, you would always have at least two Domain Controllers for redundancy and fault tolerance.

- Running DHCP/NAT on a DC: While possible, it's common practice in larger environments to separate these roles onto different servers (e.g., a dedicated member server for DHCP and a dedicated firewall/router for NAT) to improve security and performance. For a home lab, combining them on the DC is perfectly fine and efficient.

## Resources

- Building a Virtual Security Home Lab: Part 6 - Active Directory Lab Setup - Part 1 | Source Code

- How to Setup a Basic Home Lab Running Active Directory (Oracle VirtualBox) | Add Users w/PowerShell

- Home Lab Beginners guide (Hardware)

- Building a Security Lab in VirtualBox.

- r/Homelab